Description of data and processing

Figure 1: SAXS and WAXS data of SC lipid in a temperature cycle in dry and hydrated conditions. The SAXS and WAXS experiments were performed using in-house X-ray setup, GANESHA 300 XL SAXS system (JJ-Xray, Denmark). The scattering data were recorded for about 30 minutes at various selected temperatures in the heating (32, 45 and 68 °C) and cooling (45 and 32 °C) cycles. Data plotting and processing using inhouse Matlab code.

Figure 2: Bruker 500 solid-state NMR data were processed and plotted using inhouse MATLAB code derived from matNMR as described in the M&M. The data folders numbered with 5, 6, 7 contain data from direct polarization (DP), cross polarization (CP), insensitive nuclei enhanced by polarization transfer (INEPT) experiments on natural abundance 13C respectively. The proton (1H) NMR experiment is in the folder 4 and 8.

Figure 3: Spatially resolved high-resolution SAXS and WAXS characterization of the SC lipid films in capillary cells performed at the X12SA (cSAXS) beamline at the Swiss Light Source, Paul Scherrer Institut (PSI). Data processing with inhouse Matlab and Python codes.

Figure 4: Spatially and time resolved SAXS and WAXS data of SC- and DPPC lipid films in capillary cells dipped into water droplet. The characterization is performed at the X12SA (cSAXS) beamline at the Swiss Light Source, Paul Scherrer Institut (PSI). Data processing with inhouse Matlab and Python codes.

Figure S1: Same as Figure 2.

Figure S2: All the 500 solid-state NMR data were processed using inhouse MATLAB code derived from matNMR as described in the M&M. The data were exported as text file and later plotted using python scripting language in the Anaconda package.

Figure S3: Same as Figure 2. Here shown a zoomed region

Figure S4: Spatially resolved high-resolution SAXS and WAXS characterization of the SC lipid films formed at different conditions in the capillary cells. The characterization was performed at the X12SA (cSAXS) beamline at the Swiss Light Source, Paul Scherrer Institut (PSI) and at ForMAX beamline at MAXIV laboratory in Lund, Sweden. Data processing with inhouse Matlab and Python codes.

Figure S5: Dynamic light scattering (DLS) data of SC lipid dispersion when keep at 42°C and later 25°C over time. Data plotted with inhouse Matlab code.

Figure S6: Same as Figure 1.

Figure S7: Bruker 500 solid-state NMR data were processed using the website nmrium.com and exported as ascii file for later plotting using python scripting language in the Anaconda package.

Figure S8: Same as Figure 4.

Figure S9: Same as Figure 3 and Figure 4.